KT-G3 Relay Switch Firmware Upgrade Guide

1. Connect relay switch to computer via USB cable, run .EXE computer software, click [CONNECT], Enter [UPGRADE] page

2. Hold on the [SET] button on PCB board, click [RESTART] within 3 seconds, software remind you "Restart ok!",

now release the [SET] button, computer now will disconnect USB connection, the [STATUS] led light on the board will blink slowly every one second. if you didn't see led light blink slowly, then repeat this step.

3. Click [SYSTEM] back to software home page, Click [CONNECT] again, enter [UPGRADE] page, click [open firmware], choose the [kt_g3_20190403.bin] firmware file, click [Upgrade], later you will see "Upgrade success!"

4. Connect relay to computer again, run the pc software, click [CONNECT], right now you can see the firmware version is the latest version now.